2017 Digital IC Design

Homework 4: Edge-Based Line Average interpolation

| NAME |  | 沈育同 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Student ID |  | P76061386 |  |  |  |  |  |  |  |
| Simulation Result |  |  |  |  |  |  |  |  |  |
| Test pattern 1 | Functional simulation |  | Pass | Gate-level simulation |  |  | Pass | Gate-level simulation time | 2433.740 ns |
| Test pattern 2 | Functional simulation |  | Pass | Gate-level simulation |  |  | Pass | Gate-level simulation time | 2433.740 ns |
| Synthesis Result |  |  |  |  |  |  |  |  |  |
| Total logic elements |  |  |  |  | 341 |  |  |  |  |
| Total memory bit |  |  |  |  | 0 |  |  |  |  |
| Embedded multiplier 9-bit element |  |  |  |  | 0 |  |  |  |  |
| Description of your design |  |  |  |  |  |  |  |  |  |
| FSM設定： 本電路使用兩種狀態，初始化為狀態0，故只需1bit儲存current state。 Register File存取資料模式： 本電路使用的Register File共19個，儲存in_data分為兩種模式： 模式1為將in_data放置到data[18]，然後把所有data往前位移。 模式2為將data[3]放置到data[18]，然後把data[4]~data[18]往前位移。 FSM運作： 狀態0時，Register File採用模式1推進資料，out_data不斷的輸出data[18]資料。 狀態1分為兩部分： 前16 cycle，Register File採用模式1推進資料，計算out_data方式為下圖。 故本電路採用下圖電路切割pipeline，其中data[3]與data[2]之間的MUX是為解決Register File推進資料模式改變後的第1個cycle造成資料位置差異而設立。 後16 cycle，Register File採用模式2推進資料，out_data不斷的輸出data[16](因配合pipeline設計，in_data與out_data存在2 cycle delay，故需取data[16])資料。 |  |  |  |  |  |  |  |  |  |

Scoring = (Total logic elements + total memory bit + 9*embedded multiplier 9-bit element) × (longest gate-level simulation time in ns)

= (341 + 0 + 9 * 0) * 2,433.74 = 829,905